23 Tivoli Terrace South

Dun Laoghaire

Co.Dublin

A96 E622

23rd September 2026

To: The Planning Authority

Re: Appeal/Objecting to the Granting of Planning Permission for Air-Source Heat Pump

Reference number of planning application D26A/0063/WEB

Decision date 27 Aug 2026

Dear Sir/Madam,

We wish to formally appeal and object to the decision to grant planning permission for the installation and operation of the air-source heat pump at the neighbouring property.

Our concerns relate principally to (1) the noise generated by the heat pump and associated infrastructure, (2) the methodology used to assess that noise, and (3) the impact that the installation has had on our property since it became operational.

### 1. Noise and Vibration Experienced Since Installation

We are concerned that the heat pump has, at times, been operating at noise levels which appear to exceed the applicable limits. We included these observations in our original objection. The high noise levels continued at times after the servicing that was supposed to fix it. We also have significant concerns regarding the absence of adequate reassurance that the unit will be operated responsibly and within the relevant noise requirements on an ongoing basis. (The heat pump created a serious noise nuisance with a serious tonal quality from the first day it was turned on but this was not addressed for nearly 2 months despite our requests).

In our original observation on March 2026 we referred to the negative impact on sleep of the excessive noise since the installation of the heat pump adjacent to our living space on December 2025 with the most recent recording at that time being “51.7 dB(A) recorded at 2.27am on 4th March 2026. (As noted, this is well above the recommended max. levels during night-time hours; it should be less than the range of 40-45 dB A depending on the guideline used). During the summer the unit has not been running as much although we remained concerned that the unit has not been fixed as stated e.g.a reading of 51.7 dB (A) was made at our boundary wall on 8th September 2026 at 9:21pm.

In the acoustic report supplied by our neighbours the statement “It is Allegro Acoustics understanding that the heat pump will not run during night time hours.” (page 7) is particularly concerning given that heat pumps typically run during night time and this heat pump was run through the night all last winter. As we head into colder weather, it is clear the heat pump will be used more and we are concerned there are no safeguards against the noise disturbance that was so detrimental to our family’s health last winter (when we were often woken by it numerous times a night and for a time could not use our main bedroom while we looked for ways to manage the situation) as reported in our original objection.

In addition, the installation of the heat pump has coincided with the onset of a persistent vibration/noise of a prominent 50 Hz peak through the shared party wall. We have had to mask this noise disturbance using brown noise for over eight months now which has obviously been a source of ongoing stress. We have put a lot of time and resources into investigating this. With the help of a consultant sound engineer, we used a contact microphone and took readings in every room of the house to help identify where the vibration is most strongly transmitted. We have communicated to the neighbours that the clearest reading of this vibration was taken in the downstairs back room, approximately 1.5m from the floor on the party wall and this would need to be investigated from their side of the party wall but have had no reply.

As this vibrating noise coincides with the installation of the heat pump it may be part of this heat pumps infrastructure travelling through pipework or beams to the party wall and needs to be investigated and fixed. We therefore believe that it would be inappropriate to dismiss this issue without an independent investigation capable of determining whether there is any structural or low-frequency transmission associated with the heat pump or its installation.

We would therefore ask that any associated machinery and their installation are specifically investigated for a source of 50 Hz mechanical vibration.

### 2. Concerns Regarding the Allegro Acoustic Assessment

We have reviewed the Allegro report commissioned to investigate the noise and have identified a number of technical issues which, in our view, raise questions regarding the robustness of its methodology and conclusions.

#### Survey Methodology

The assessment relies primarily on unattended noise monitoring, which necessarily requires assumptions to be made regarding prevailing noise sources and environmental conditions.

In this particular case, attended monitoring would have been preferable. This would have enabled a suitably qualified acoustician to make observations during the measurements and would provide greater confidence in interpreting the measured data.

The report itself acknowledges that demolition works, construction activity, wind and rustling foliage influenced the measurements. These factors raise questions as to whether the recorded data can reliably be regarded as representative of the heat pump's normal operating conditions.

#### Applicability of the Guidance Used

The report states that the survey was undertaken in accordance with EPA NG4 guidance. However, NG4 is principally intended for EPA-licensed industrial activities rather than domestic air-source heat pumps.

Clarification should therefore be provided as to why NG4 was considered the appropriate guidance for assessing this residential heat pump installation and whether the assessment methodology adequately reflects the specific planning and residential context of the installation.

#### Noise Monitor Position

Section 4.2.1 of the report states that the sound level meter was mounted at 1.5 metres above ground level and at least 3.5 metres away from any sound-reflecting surfaces.

However, Figures 1, 2 and 4 appear to indicate that the monitoring equipment was positioned adjacent to the dwelling façade. The report also states that the monitoring location was approximately 2 metres from the neighbouring bedroom window/façade.

There therefore appears to be an inconsistency between the stated methodology and the apparent monitoring location.

Clarification should be sought regarding the precise location of the monitoring equipment and whether its position complied with the methodology stated in the report. This is particularly important given the proximity to the neighbouring bedroom and the potential effect of reflected sound on the measurements.

#### Meteorological Conditions

Section 4.6 states that, during the monitoring period of 25/06/2026 to 26/06/2026, average wind speeds remained below 8 m/s, while gusts exceeded 8 m/s.

Environmental noise measurements are generally expected to be undertaken under meteorological conditions suitable for obtaining reliable measurements, with average wind speeds typically below 5 m/s.

It is therefore unclear whether any data potentially affected by elevated wind speeds or gust conditions were excluded from the analysis.

Clarification should be provided regarding the quality-assurance procedures applied to the dataset and whether measurements taken during unsuitable or potentially influential weather conditions were removed before the conclusions were reached.

#### Planning Exemption Noise Criterion

Of particular concern is the apparent absence of any specific reference in the report to Part 1 of Schedule 2 of the Planning and Development Regulations 2001, Class 2(d) in relation to air-source heat pumps.

The assessment does not explicitly address the requirement concerning noise levels at the nearest neighbouring inhabited dwelling, including the relevant 43 dB(A) and background-noise criteria.

Given that this is a residential installation, the relevance of this planning criterion should be explicitly considered and compliance demonstrated.

We consider this omission significant because it is directly relevant to determining whether the installation can operate without causing unacceptable noise impacts to neighbouring residential properties.

#### Selection of Assessment Periods

The report reduces the assessment to two selected 15-minute periods which were considered to be the least affected by external sources.

While shorter assessment periods may be appropriate where they are demonstrably representative, the basis for selecting these particular periods is not sufficiently clear, particularly given that the measurements were obtained through unattended monitoring.

Further justification should therefore be provided as to how these periods were identified, why they were considered representative of normal heat pump operation, and how this was verified.

#### Heat Pump Operational Behaviour

Figures 6 and 7 show variations in the measured sound levels during transitions between different heat pump operational states.

However, the report does not adequately explain these variations or establish whether they correspond to start-up, shut-down, ramp-down or other normal operational behaviour.

If these changes occur during the normal operation of the heat pump, they should form part of the assessment rather than being treated as incidental or excluded from consideration.

It is important that the assessment reflects the actual operating behaviour of the equipment over a normal cycle, rather than relying only upon selected periods of operation.

#### Night-Time Background Noise

The report records a night-time background noise level of approximately 23 dB.

This represents a very low night-time acoustic environment. However, the report does not adequately discuss the sensitivity of the receiving environment or the significance of operating mechanical equipment against such a low background level.

In a quiet residential environment, a relatively modest mechanical or tonal noise source may be perceptible to neighbouring residents, particularly during night-time periods.

Further consideration should therefore be given to receptor sensitivity and the implications of such low background noise levels.

#### Qualified Acoustician Observations

The report concludes that no tonal, intermittent or impulsive characteristics were identified, apparently relying to a significant extent upon observations.

Given that the survey was primarily unattended, it is unclear how these observations could have been made reliably throughout the entire monitoring period.

Attended monitoring and contemporaneous observations by a suitably qualified acoustician would provide greater confidence in determining whether tonal, intermittent or other characteristics were present during the heat pump's operation.

### Overall Concerns Regarding the Allegro Report

Taken together, we consider that the following issues are particularly significant:

- The apparent inconsistency regarding the location of the noise monitoring equipment.

- The potentially unsuitable or variable meteorological conditions during the monitoring period.

- The reliance upon unattended rather than attended monitoring.

- The absence of explicit consideration of the applicable Irish planning noise criterion for air-source heat pumps.

- The limited assessment of the heat pump's different operational states.

- The selection of specific 15-minute periods without sufficient justification as to their representativeness.

- The lack of adequate consideration of the very low night-time background noise level.

- The absence of a satisfactory investigation into the reported tonal/vibrational disturbance experienced through the shared party wall.

In our view, these matters raise legitimate concerns about whether the Allegro assessment provides a sufficiently robust basis upon which to conclude that the installation will not adversely affect neighbouring residential amenity.

We also understand that Dún Laoghaire-Rathdown County Council previously raised substantive concerns regarding the original acoustic report. It would therefore be reasonable to expect that those concerns would have been adequately addressed in any revised assessment. We remain concerned that similar methodological and reporting deficiencies may still be present.

### Request for Further Consideration

In light of the above, we respectfully request that the decision to grant planning permission be reconsidered.

At a minimum, we believe that a further independent acoustic assessment should be undertaken by an appropriately qualified acoustician. This assessment should include:

- Attended as well as unattended monitoring;

- Assessment of the heat pump throughout its normal operating cycle;

- Specific consideration of start-up, shut-down and ramping behaviour;

- Appropriate night-time monitoring;

- Assessment against the relevant Irish planning noise requirements;

- Confirmation that the monitoring equipment is positioned correctly and in accordance with the stated methodology;

- Proper consideration of meteorological conditions and data quality;

- Assessment of tonal and low-frequency noise;

- Investigation of the approximately 50 Hz vibration experienced through the party wall; and

- Consideration of the impact upon the neighbouring inhabited dwelling, particularly the bedroom area.

(3) The impact that the installation has had on our property since it became operational.

We are not seeking to prevent the reasonable use of renewable heating technology. We understand that to encourage the installation of heat pumps the guidance has changed as to where they can be installed on a property. Our neighbours, who have both a side passage and back garden, chose to install theirs to the front of the property away from their main living space and main bedroom. Unfortunately it is therefore closer to our main living room and main bedroom and has historically caused serious noise nuisance in both these rooms and may be the cause (through its associated infrastructure) of the persistent vibrating hum present in all rooms with the shared party wall which include a child’s bedroom.

This heat pump has been run at above recommended noise levels. The neighbours have very briefly mentioned there was an issue with the installation. We have concerns over the acoustic report provided and whether as winter approaches and the heat pump operates more extensively what effect this will have on the amenity of our home.

Our concern is that the installation and location should operate in a manner that does not cause unreasonable noise or vibration to neighbouring residents and that compliance with the relevant planning requirements should be objectively demonstrated. If the heat pump persists in being above recommended noise levels this winter (this was outlined as no more than 5 decibels above background noise and we are presuming with no tonal interference) we do not know who is responsible for enforcing the recommendations.

Given the concerns outlined above, we respectfully request that the acoustic evidence supporting the grant of planning permission be reviewed and that appropriate consideration be given to the impact of the installation on our family’s residential amenity.

Yours faithfully,

Grace and John Greaney